-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-==-

-                                                          -

-                     Card-o-Matic                         -

-               Magnetic Strip Analyzer                    -

-                                                          -

-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-=-==-

Card-o-matic is a magnetic strip card reader interface, bit

analyzer, and editor.  The program comes loaded with tables

which can decode the 7 bit and 5 bit ANSI standards. Other

functions available are decryption, generation, and cut or

pasting of bit streams.

Commands available:

I/O functions:

-----------------------------------------------------------

Key                    Command

r                 Toggle reader ON/OFF

l                 Load a card bit pattern. (Enter '?' for a

directory at any input where a file is

required)

s                 Save a card bit pattern

p                 Edit parameters

-lookup table

-default lookup table

-LPT port address

<F1>              Help file

q                 Quit program

Analysis functions:

-----------------------------------------------------------

Key                    Command

1-4               Change to window #?

a                 Analyze and decode bit pattern

c                 Chomp-bit analysis

d                 Decrypt code

h                 Highlight area interest marked

<F5>              Show (in text) bit pattern of highlighted

area

Ctrl-L            Redraws the current window

Cut and Paste functions:

-----------------------------------------------------------

Key                    Command

L or R arrows     Mark location of paste/ or Highlight

<F2>              Copy block into memory buffer

<DEL> or <F3>     Copy into block and delete block

<INS> or <F4>     Insert block

Read card:

To read a card, hit <r> to toggle the option to ON

then swipe the card through the reader. To prevent accidental

erasure of data, hit <r> to toggle back to OFF

After swiping the card, a graphical display of the bits are plotted

across the screen. The decoded message is also displayed at the bottom.

Loading and Saving:

Type in the name of the file WITHOUT the extension. To see a

directory files, type "?" and press return.

Parameter change:

To skip to different parameters, press the TAB button.

To change the parameter hit a key, and then type in the

data.

Pattern file: change current pattern file

Default Pattern file: pattern loaded when program starts

LPT Port: the address of your printer port

if you don't know the address of your port, try

888 or 632. Find out by using MSD.EXE in the \WINDOWS

directory.

HINT! You can also use the F7 and F8 keys to change the bit pattern

between ANSI7Bit and ANSI5Bit formats!

Help screen:

Hit F1. You're looking at the help file.

Analyze bit pattern:

If the format of the card strip is known, the data can be

decoded by hitting the "a" button. An explanation of the

decoding method follows:

At the beginning of the program, a file is loaded which

contains the lookup table used to compare the data.

To format of the file is:

Name of file:  ???.pat  as in   ansi7b.pat

Contents of file:

A

1000011

B

0100011

C

1100010

D

0010011

E

1010010

F

0110010

G

1110011

"A" is the character and 1000011 is the word which represents

the character.

eg.

If the magnetic bit was 100001111000100100011

the the decoded data would be ABC

The word size must be the same for all characters.

The word size for the previous example is 7 bits.

CARDOMATIC comes with the ANSI 7-bit and the ANSI 5-bit patterns.

The files are called ANSI7B.PAT and ANSI5B.PAT

You can create your own pattern files by creating a file with

the .pat extension and then entering the data in the above format.

Highlight block

If the area marked is not highlighted, hitting "h" will

highlight the block.

Redraw window

If the window becomes too cluttered with junk, hit Ctrl-L will

redraw the window.

Decrypt!

If the format of the card is not known, the bit format and patterns

could possibly be decrypted with the "d" function. If the whole code

can not be decrypted, it is likely that you will get clues as to the

word size and a few matches between your characters and the mag-strip

bit patterns!

To use the decrypt function, press "d" and enter some data that may

be known to be on the card. The input is CaSe SeNsiTiVe, so john_doe

is not the same as JOHN_DOE. The program will search through and attempt

to find a pattern match between your input and the mag-strip bit array.

If a match is found, a window of text will appear at the bottom

displaying where the match begins, ends, and the bit patterns that

match each character.  Use the arrow keys to scroll up and down the

window.  If a further search is desired, hit the <enter> key.  If

you believe that the match is correct, hit <esc> to return to the main

menu.  The beginning and end points are now loaded into memory. If you

wish to copy the data into the buffer, hit <F2> to copy. If you wish to

delete the block, hit the <DEL> key.

Some knowledge of the information on the card is required

in order to decrypt. The program will look at your at your input and

generalize each character into a pattern. It will then take the bit

array from the card and then divide that into "words" of the same

bit size.  Then, the words are generalized into patterns. The pattern

array from the given input is then compared with the pattern array

generated from the mag-strip data. If no match is found, the comparison

is shifted over one word with reference to the mag-strip pattern array.

This process is loop'ed beginning with 12 bit word sizes down to 1 bit.

The decryption algorithm is a little complex and will not be discussed

further in this help file. If more information is desired, send

E-mail to rxy5310@hertz.njit.edu.

Hint: It is more likely for a true match to be found if the input data

has multiple characters which are the same. (eg. BABOONOGRAM which

includes 4 "o"'s is more likely to find a true match than MAPS which

has no matching letters)

To become familiar with the function, load up your own credit card

data and search for your name.

Chomp-bit Analysis

Sometimes, the data read from the card may not be accurate, or an odd

word-length header may precede the data on the card. For cases like this,

hit the "c" button from the main menu to analyze the pattern.  Chomp bit

analysis will remove one bit from the front of the card and then decode

the card using the current lookup pattern table.  The data will be displayed

on the bottom window. Use the arrow keys to scroll up and down.

This function may also be useful in observing data taken from portions of

a broken or damaged card.

Generate bit code

If you wish to generate a bit array, hit the "g" button from the main menu

and type in a line of text. Characters which are not found in the lookup

table will be substituted with a word of 0's. If you wish to enter

a bit pattern which is not in the lookup table, type in a "~" followed

by the bit pattern in the form of 0's and 1's. End the pattern with another

"~"

For example:

%B12345^DOE/JOHN___~0010110100101~THATS ALL

The 0010110100101 will be inserted between the bit pattern for

DOE/JOHN___ and THATS ALL.  You can type in a whole bit array with

the ~ option, but why?

Cut and paste functions:

Use the arrow keys to move the marker to the beginning of your block

hit <enter> to mark it.  Move the marker to the end point, then hit

<enter> again. If you wish to go directly to a certain point, hit the

<SPACE> bar and then type in the location. This could be quicker than

using the arrow keys to move the marker.

Delete block:

To delete the block, mark a block, then hit <del>

Insert block:

To insert a block, move the marker to the point at which you wish to

insert the block.  Then hit the <insert> key.

Copy block:

If a block is marked (such as when using the Decrypt function),

you can copy the block into the memory buffer by hitting the <F2> key.

Display buffer:

If you would like to see the contents of the buffer, hit the <F5> button.

Troubleshooting:

Some TSR's such as Power Manager found on laptops may affect the

parallel port and thus cause Card-O-Matic to malfunction.

If the program does not work initially, change the LPT parameter

to the address of your port. If you do not know your port address,

try to find out by using MSD.EXE in the windows directory.

For additional information, write to Rolan Yang

email address:    rxy5310@hertz.njit.edu

=============================================================================

Tips on analyzing cards:

If the format of the card is not known, some information about the

physical pattern on the strip can be obtained by sprinkling fine iron

fillings on to the strip and then lightly tapping the card. The fillings

will stick to the strip at the locations where there is magnetic polarity.

The pattern can be preserved on a piece of cellophane tape.

=============================================================================

A file called EX.BAS is provided. This is an example program that will

fetch the data from a swipe card and display it to the screen in decoded

ANSI 7 bit character format. If you are using under a 486-25 or slower,

the basic compiler may be too slow to capture the information. In order

to accurately capture data from the reader, you will need a BASIC compiler

which will speed the execution of the program.

The following file was not written by me. It is provided as additional

information which may further educate you about the composition of a

magnetic strip card.

Phrack magzine archives can be found at   freeside.com

From PHRACK issue#37

*******************************************************************************

*                                                                             *

*              Card-O-Rama: Magnetic Stripe Technology and Beyond             *

*                                                                             *

*                                  or                                         *

*                                                                             *

*                  "A Day in the Life of a Flux Reversal"                     *

*                                                                             *

*                                                                             *

*                                                                             *

*                     by: ..oooOO Count Zero OOooo.. .RDT.       11/22/91     *

*                                                                             *

*******************************************************************************

---A production of :    -=Restricted -=Data -=Transmissions    :

:                                           :

: "Truth is cheap, but information costs."  :

Look in your wallet.  Chances are you own at least 3 cards that have

magnetic stripes on the back.  ATM cards, credit cards, calling cards,

frequent flyer cards, ID cards, passcards,...cards, cards, cards!  And chances

are you have NO idea what information is on those stripes or how they are

encoded.  This detailed document will enlighten you and hopefully spark your

interest in this fascinating field.  None of this info is 'illegal'...but

MANY organizations (government, credit card companies, security firms,

etc.) would rather keep you in the dark.  Also, many people will IMMEDIATELY

assume that you are a CRIMINAL if you merely "mention" that you are

"interested in how magnetic stripe cards work."  Watch yourself, ok?  Just

remember that there's nothing wrong with wanting to know how things work, altho

in our present society, you may be labelled a "deviant" (or worse, a <gasp>

"hacker!").

Anyway, I will explain in detail how magstripes are encoded and give

several examples of the data found on some common cards.  I will also cover the

technical theory behind magnetic encoding, and discuss magnetic encoding

alternatives to magstripes (Wiegand, barium ferrite).  Non-magnetic card

technology (bar code, infrared, etc.) will be described.  Finally, there will

be an end discussion on security systems and the ramifications of emergent

"smartcard" and biometric technologies.

*DISCLAIMER*

Use this info to EXPLORE, not to EXPLOIT.  This text is presented for

informational purposes only, and I cannot be held responsible for anything you

do or any consequences thereof.  I do not condone fraud, larceny, or any other

criminal activities.

*A WARNING*

I've noticed lately a few "books" and "magazines" for sale that were

FILLED with PHILES on a variety of computer topics.  These philes were

originally released into the Net with the intention of distributing them for

FREE.  HOWEVER, these philes are now being PACKAGED and sold FOR PROFIT.  This

really pisses me off.  I am writing this to be SHARED for FREE, and I ask no

payment.  Feel free to reprint this in hardcopy format and sell it if you must,

but NO PROFITS must be made.  Not a fucking DIME!  If ANYONE reprints this

phile and tries to sell it FOR A PROFIT, I will hunt you down and make your

life miserable.  How?  Use your imagination.  The reality will be worse.

**  MAGSTRIPE FIELDS, HEADS, ENCODING/READING **

Whew!  I'll get down to business now.  First, I am going to explain the

basics behind fields, heads, encoding and reading.  Try and absorb the THEORY

behind encoding/reading.  This will help you greatly if you ever decide to

build your own encoder/reader from scratch (more on that later).

FERROMAGNETIC materials are substances that retain magnetism after an external

magnetizing field is removed.  This principle is the basis of ALL magnetic

recording and playback.  Magnetic POLES always occur in pairs within magnetized

material, and MAGNETIC FLUX lines emerge from the NORTH pole and

terminate at the SOUTH.  The elemental parts of MAGSTRIPES are ferromagnetic

particles about 20 millionths of an inch long, each of which acts like a tiny

bar magnet.  These particles are rigidly held together by a resin binder.

The magnetic particles are made by companies which make coloring pigments

for the paint industry, and are usually called pigments.  When making the

magstripe media, the elemental magnetic particles are aligned with their

North-South axes parallel to the magnetic stripe by means of an external

magnetic fields while the binder hardens.

These particles are actually permanent bar magnets with TWO STABLE

POLARITIES.  If a magnetic particle is placed in a strong external magnetic

field of the opposite polarity, it will FLIP its own polarity (North becomes

South, South becomes North).  The external magnetic field strength required to

produce this flip is called the COERCIVE FORCE, or COERCIVITY of the particle.

Magnetic pigments are available in a variety of coercivities (more on that

lateron).

An unencoded magstripe is actually a series of North-South magnetic

domains (see Figure 1).  The adjacent N-S fluxes merge, and the entire stripe

acts as a single bar magnet with North and South poles at its ends.

Figure 1:               N-S.N-S.N-S.N-S.N-S.N-S.N-S.N-S <-particles in stripe

---------

represented as-> N-----------------------------S

However, if a S-S interface is created somewhere on the stripe, the fluxes

will REPEL, and we get a concentration of flux lines around the S-S interface.

(same with N-N interface)  ENCODING consists of creating S-S and N-N

interfaces, and READING consists of (you guessed it) detecting 'em.  The S-S

and N-N interfaces are called FLUX REVERSALS.

||| ||| <-flux lines

Figure 2:      N------------N-N-S-S-----------------S

---------     flux lines -> ||| |||

The external magnetic field used to flip the polarities is produced by a

SOLENOID, which can REVERSE its polarity by reversing the direction of CURRENT.

An ENCODING head solenoid looks like a bar magnet bent into the shape of a ring

so that the North/South poles are very close and face each other across a tiny

gap.  The field of the solenoid is concentrated across this gap, and when

elemental magnetic particles of the magstripe are exposed to this field, they

polarize to the OPPOSITE (unlike poles attract).  Movement of the stripe past

the solenoid gap during which the polarity of the solenoid is REVERSED will

produce a SINGLE flux reversal (see Figure 3).  To erase a magstripe, the

encoding head is held at a CONSTANT polarity and the ENTIRE stripe is moved

past it.  No flux reversals, no data.

| |  <----wires leading to solenoid

| |       (wrapped around ring)

/-|-|-\

/       \

Figure 3:                  |       | <----solenoid (has JUST changed polarity)

---------                  \       /

\ N S / <---gap in ring.. NS polarity across gap

N----------------------SS-N-------------------------S

^^

<<<<<-direction of stripe movement

S-S flux reversal created at trailing edge of solenoid!

So, we now know that flux reversals are only created the INSTANT the

solenoid CHANGES its POLARITY.  If the solenoid in Figure 3 were to remain at

its current polarity, no further flux reversals would be created as the

magstripe moves from right to left.  But, if we were to change the solenoid

gap polarity from NS to *SN*, then (you guessed it) a *N-N* flux reversal would

instantly be created.  Just remember, for each and every reversal in solenoid

polarity, a single flux reversal is created (commit it to memory..impress your

friends..be a tech weenie!).  An encoded magstripe is therefore just a series

of flux reversals (NN followed by SS followed by NN ...).

DATA! DATA! DATA!  That's what you want!  How the hell are flux reversals

read and interpreted as data?  Another solenoid called a READ HEAD is used to

detect these flux reversals.  The read head operates on the principle of

ELECTROMAGNETIC RECIPROCITY: current passing thru a solenoid produces a

magnetic field at the gap, therefore, the presence of a magnetic field at the

gap of a solenoid coil will *produce a current in the coil*!  The strongest

magnetic fields on a magstrip are at the points of flux reversals.  These are

detected as voltage peaks by the reader, with +/- voltages corresponding to

NN/SS flux reversals (remember, flux reversals come in 2 flavors).

See Figure 4.

magstripe---> -------NN--------SS--------NN---------SS------

Figure 4:     voltage-----> .......+.........-.........+...........-.....

---------

----------          -------------

peak readout-->        |        |          |           |

--------|        |----------|           |----

The 'peak readout' square waveform is critical.  Notice that the voltage

peak remains the same until a new flux reversal is encountered.

Now, how can we encode DATA?  The most common technique used is known as

Aiken Biphase, or 'two-frequency coherent-phase encoding' (sounds impressive,

eh?).  First, digest the diagrams in Figure 5.

Figure 5:       ----------        ----------        ----------

---------       |        |        |        |        |        |  <- peak

a)     |        |--------|        |--------|        |     readouts

*   0    *   0    *   0    *   0    *   0    *

-----    -----    -----    -----    -----    -

|   |    |   |    |   |    |   |    |   |    |

b)      |   |----|   |----|   |----|   |----|   |----|

*   1    *   1    *   1    *   1    *   1    *

-----    ----------        -----    -----    -

|   |    |        |        |   |    |   |    |

c)      |   |----|        |--------|   |----|   |----|

*   1    *   0    *   0    *   1    *   1    *

There ya have it.   Data is encoded in 'bit cells,' the frequency of which

is the frequency of '0' signals.  '1' signals are exacty TWICE the frequency

of '0' signals.  Therefore, while the actual frequency of the data passing

the read head will vary due to swipe speed, data density, etc, the '1'

frequency will ALWAYS be TWICE the '0' frequency.  Figure 5C shows exactly how

'1' and '0' data exists side by side.

We're getting closer to read DATA!  Now, we're all familiar with binary

and how numbers and letters can be represented in binary fashion very easily.

There are obviously an *infinite* number of possible standards, but thankfully

the American National Standards Institute (ANSI) and the International

Standards Organization (ISO) have chosen 2 standards.  The first is

** ANSI/ISO BCD Data format **

This is a 5-bit Binary Coded Decimal format.  It uses a 16-character set,

which uses 4 of the 5 available bits.  The 5th bit is an ODD parity bit, which

means there must be an odd number of 1's in the 5-bit character..the parity bit

will 'force' the total to be odd.  Also, the Least Significant Bits are read

FIRST on the strip.  See Figure 6.

The sum of the 1's in each case is odd, thanks to the parity bit.  If the

read system adds up the 5 bits and gets an EVEN number, it flags the read

as ERROR, and you gotta scan the card again.  (yeah, I *know* a lot of you

out there *already* understand parity, but I gotta cover all the bases...not

everyone sleeps with their modem and can recite the entire AT command set

at will, you know ;).  See Figure 6 for details of ANSI/ISO BCD.

Figure 6:        ANSI/ISO BCD Data Format

---------

* Remember that b1 (bit #1) is the LSB (least significant bit)!

* The LSB is read FIRST!

* Hexadecimal conversions of the Data Bits are given in parenthesis (xH).

--Data Bits--   Parity

b1  b2  b3  b4   b5    Character  Function

0   0   0   0    1        0 (0H)    Data

1   0   0   0    0        1 (1H)      "

0   1   0   0    0        2 (2H)      "

1   1   0   0    1        3 (3H)      "

0   0   1   0    0        4 (4H)      "

1   0   1   0    1        5 (5H)      "

0   1   1   0    1        6 (6H)      "

1   1   1   0    0        7 (7H)      "

0   0   0   1    0        8 (8H)      "

1   0   0   1    1        9 (9H)      "

0   1   0   1    1        : (AH)    Control

1   1   0   1    0        ; (BH)    Start Sentinel

0   0   1   1    1        < (CH)    Control

1   0   1   1    0        = (DH)    Field Separator

0   1   1   1    0        > (EH)    Control

1   1   1   1    1        ? (FH)    End Sentinel

***** 16 Character 5-bit Set *****

10 Numeric Data Characters

3 Framing/Field Characters

3 Control Characters

The magstripe begins with a string of Zero bit-cells to permit the

self-clocking feature of biphase to "sync" and begin decoding.

A "Start Sentinel" character then tells the reformatting process where to start

grouping the decoded bitstream into groups of 5 bits each.  At the end of the

data, an "End Sentinel" is encountered, which is followed by an "Longitudinal

Redundancy Check (LRC) character.  The LRC is a parity check for the sums of

all b1, b2, b3, and b4 data bits of all preceding characters.  The LRC

character will catch the remote error that could occur if an individual

character had two compensating errors in its bit pattern (which would fool the

5th-bit parity check).

The START SENTINEL, END SENTINEL, and LRC are collectively called "Framing

Characters", and are discarded at the end of the reformatting process.

** ANSI/ISO ALPHA Data Format **

Alphanumeric data can also be encoded on magstripes.  The second ANSI/ISO

data format is ALPHA (alphanumeric) and involves a 7-bit character set with

64 characters.  As before, an odd parity bit is added to the required 6 data

bits for each of the 64 characters.  See Figure 7.

Figure 7:

---------             ANSI/ISO ALPHA Data Format

* Remember that b1 (bit #1) is the LSB (least significant bit)!

* The LSB is read FIRST!

* Hexadecimal conversions of the Data Bits are given in parenthesis (xH).

------Data Bits-------   Parity

b1  b2  b3  b4  b5  b6     b7    Character   Function

0   0   0   0   0   0     1      space (0H) Special

1   0   0   0   0   0     0        ! (1H)      "

0   1   0   0   0   0     0        " (2H)      "

1   1   0   0   0   0     1        # (3H)      "

0   0   1   0   0   0     0        $ (4H)      "

1   0   1   0   0   0     1        % (5H)   Start Sentinel

0   1   1   0   0   0     1        & (6H)   Special

1   1   1   0   0   0     0        ' (7H)      "

0   0   0   1   0   0     0        ( (8H)      "

1   0   0   1   0   0     1        ) (9H)      "

0   1   0   1   0   0     1        * (AH)      "

1   1   0   1   0   0     0        + (BH)      "

0   0   1   1   0   0     1        , (CH)      "

1   0   1   1   0   0     0        - (DH)      "

0   1   1   1   0   0     0        . (EH)      "

1   1   1   1   0   0     1        / (FH)      "

0   0   0   0   1   0     0        0 (10H)    Data (numeric)

1   0   0   0   1   0     1        1 (11H)     "

0   1   0   0   1   0     1        2 (12H)     "

1   1   0   0   1   0     0        3 (13H)     "

0   0   1   0   1   0     1        4 (14H)     "

1   0   1   0   1   0     0        5 (15H)     "

0   1   1   0   1   0     0        6 (16H)     "

1   1   1   0   1   0     1        7 (17H)     "

0   0   0   1   1   0     1        8 (18H)     "

1   0   0   1   1   0     0        9 (19H)     "

0   1   0   1   1   0     0        : (1AH)   Special

1   1   0   1   1   0     1        ; (1BH)      "

0   0   1   1   1   0     0        < (1CH)      "

1   0   1   1   1   0     1        = (1DH)      "

0   1   1   1   1   0     1        > (1EH)      "

1   1   1   1   1   0     0        ? (1FH)   End Sentinel

0   0   0   0   0   1     0        @ (20H)   Special

1   0   0   0   0   1     1        A (21H)   Data (alpha)

0   1   0   0   0   1     1        B (22H)     "

1   1   0   0   0   1     0        C (23H)     "

0   0   1   0   0   1     1        D (24H)     "

1   0   1   0   0   1     0        E (25H)     "

0   1   1   0   0   1     0        F (26H)     "

1   1   1   0   0   1     1        G (27H)     "

0   0   0   1   0   1     1        H (28H)     "

1   0   0   1   0   1     0        I (29H)     "

0   1   0   1   0   1     0        J (2AH)     "

1   1   0   1   0   1     1        K (2BH)     "

0   0   1   1   0   1     0        L (2CH)     "

1   0   1   1   0   1     1        M (2DH)     "

0   1   1   1   0   1     1        N (2EH)     "

1   1   1   1   0   1     0        O (2FH)     "

0   0   0   0   1   1     1        P (30H)     "

1   0   0   0   1   1     0        Q (31H)     "

0   1   0   0   1   1     0        R (32H)     "

1   1   0   0   1   1     1        S (33H)     "

0   0   1   0   1   1     0        T (34H)     "

1   0   1   0   1   1     1        U (35H)     "

0   1   1   0   1   1     1        V (36H)     "

1   1   1   0   1   1     0        W (37H)     "

0   0   0   1   1   1     0        X (38H)     "

1   0   0   1   1   1     1        Y (39H)     "

0   1   0   1   1   1     1        Z (3AH)     "

1   1   0   1   1   1     0        [ (3BH)    Special

0   0   1   1   1   1     1        \ (3DH)    Special

1   0   1   1   1   1     0        ] (3EH)    Special

0   1   1   1   1   1     0        ^ (3FH)    Field Separator

1   1   1   1   1   1     1        _ (40H)    Special

***** 64 Character 7-bit Set *****

* 43 Alphanumeric Data Characters

* 3 Framing/Field Characters

* 18 Control/Special Characters

The two ANSI/ISO formats, ALPHA and BCD, allow a great variety of data to be

stored on magstripes.  Most cards with magstripes use these formats, but

occasionally some do not.  More about those lateron.

** Tracks and Encoding Protocols **

Now we know how the data is stored.  But WHERE is the data stored on the

magstripe?  ANSI/ISO standards define *3* Tracks, each of which is used for

different purposes.  These Tracks are defined only by their location on the

magstripe, since the magstripe as a whole is magnetically homogeneous.  See

Figure 8.

Figure 8:

---------          <edge of card>

_________________________________________________________________

|                  ^         ^         ^

|------------------| 0.223"--|---------|-------------------------

|                  |         | 0.353"  |            ^

|..................|.........|.........| 0.493"     |

| Track #1  0.110"           |         |            |

|............................|.........|...     <MAGSTRIPE>

|                            |         |            |

|............................|.........|...         |

| Track #2  0.110"                     |            |

|......................................|...         |

|                                      |            |

|......................................|...         |

| Track #3  0.110"                                  |

|..........................................         |

|                                                   |

|------------------------------------------------------------------

|

|                   <body of card>

|

You can see the exact distances of each track from the edge of the card, as

well as the uniform width and spacing.  Place a magstripe card in front of you

with the magstripe visible at the bottom of the card.  Data is encoded from

left to right (just like reading a book, eh?).  See Figure 9.

Figure 9:

---------          ANSI/ISO Track 1,2,3 Standards

Track     Name     Density     Format    Characters     Function

--------------------------------------------------------------------

1       IATA     210 bpi     ALPHA        79        Read Name & Account

2       ABA       75 bpi      BCD         40        Read Account

3       THRIFT   210 bpi      BCD        107        Read Account &

*Encode* Transaction

*** Track 1 Layout: ***

| SS | FC |  PAN  |   Name   | FS |  Additional Data | ES | LRC |

SS=Start Sentinel "%"

FC=Format Code

PAN=Primary Acct. # (19 digits max)

FS=Field Separator "^"

Name=26 alphanumeric characters max.

Additional Data=Expiration Date, offset, encrypted PIN, etc.

ES=End Sentinel "?"

LRC=Longitudinal Redundancy Check

*** Track 2 Layout: ***

| SS |  PAN  | FS |  Additional Data  | ES | LRC |

SS=Start Sentinel ";"

PAN=Primary Acct. # (19 digits max)

FS=Field Separator "="

Additional Data=Expiration Date, offset, encrypted PIN, etc.

ES=End Sentinel "?"

LRC=Longitudinal Redundancy Check

*** Track 3 Layout: **  Similar to tracks 1 and 2.  Almost never used.

Many different data standards used.

Track 2, "American Banking Association," (ABA) is most commonly used.  This

is the track that is read by ATMs and credit card checkers.  The ABA designed

the specifications of this track and all world banks must abide by it.  It

contains the cardholder's account, encrypted PIN, plus other discretionary

data.

Track 1, named after the "International Air Transport Association," contains

the cardholder's name as well as account and other discretionary data.  This

track is sometimes used by the airlines when securing reservations with a

credit card; your name just "pops up" on their machine when they swipe your

card!

Since Track 1 can store MUCH more information, credit card companies are trying

to urge retailers to buy card readers that read Track 1.  The *problem* is that

most card readers read either Track 1 or Track 2, but NOT BOTH!  And the

installed base of readers currently is biased towards Track 2.  VISA USA is at

the front of this 'exodus' to Track 1, to the point where they are offering

Track 1 readers at reduced prices thru participating banks.  A spokesperson for

VISA commented:

"We think that Track 1 represents more flexibility and the

potential to deliver more information, and we intend to

build new services around the increased information."

What new services?  We can only wait and see.

Track 3 is unique.  It was intended to have data read and WRITTEN on it.

Cardholders would have account information UPDATED right on the magstripe.

Unfortunately, Track 3 is pretty much an orphaned standard.  Its *original*

design was to control off-line ATM transactions, but since ATMs are now on-line

ALL THE TIME, it's pretty much useless.  Plus the fact that retailers and banks

would have to install NEW card readers to read that track, and that costs $$.

Encoding protocol specifies that each track must begin and end with a length

of all Zero bits, called CLOCKING BITS.  These are used to synch the self-

clocking feature of biphase decoding.  See Figure 10.

Figure 10:                              end sentinel

start sentinel      |  longitudinal redundancy check

|                  |  |

000000000000000 SS.................ES LRC 0000000000000000

leading           data, data, data           trailing

clocking bits                                clocking bits

(length varies)                             (length varies)

THAT'S IT!!!  There you have the ANSI/ISO STANDARDS!  Completely explained.

Now, the bad news.  NOT EVERY CARD USES IT!  Credit cards and ATM cards will

follow these standards.  BUT, there are many other types of cards out there.

Security passes, copy machine cards, ID badges, and EACH of them may use a

PROPRIETARY density/format/track-location system.  ANSI/ISO is REQUIRED for

financial transaction cards used in the international interbank network.  All

other cards can play their own game.

The good news.  MOST other cards follow the standards, because it's EASY to

follow a standard instead of WORKING to make your OWN!  Most magstripe cards

other than credit cards and ATM cards will use the same Track specifications,

and use either BCD or ALPHA formats.

** A Bit About Magstripe Equipment **

"Wow, now I know how to interpret all that data on magstripes!  But...

waitasec, what kind of equipment do I need to read the stripes?

Where can I buy a reader?  I don't see any in Radio Shack!!"

Sorry, but magstripe equipment is hard to come by.  For obvious reasons,

card readers are not made commonly available to consumers.  How to

build one is the topic for another phile (and THIS phile is already too long!).

Your best bets are to try and scope out Electronic Surplus Stores and flea

markets.  Don't even bother trying to buy one directly from a manufacturer,

since they will immediately assume you have "criminal motives."  And as for

getting your hands on a magstripe ENCODER...well, good luck!  Those rare

beauties are worth their weight in gold.  Keep your eyes open and look around,

and MAYBE you'll get lucky!  A bit of social engineering can go a LONG way.

There are different kinds of magstripe readers/encoders.  The most common

ones are "swipe" machines: the type you have to physically slide the card thru.

Others are "insertion" machines: like ATM machines they 'eat' your card, then

regurgitate it after the transaction.  Costs are in the thousands of dollars,

but like I said, flea markets and surplus stores will often have GREAT deals

on these things.  Another problem is documentation for these machines.  If you

call the manufacturer and simply ask for 'em, they will probably deny you the

literature. "Hey son, what are you doing with our model XYZ swipe reader?  That

belongs in the hands of a 'qualified' merchant or retailer, not some punk kid

trying to 'find out how things work!"  Again, some social engineering may be

required.  Tell 'em you're setting up a new business.  Tell 'em you're working

on a science project.  Tell 'em anything that works!

2600 Magazine recently had a good article on how to build a machine that

copies magstripe cards.  Not much info on the actual data formats and encoding

schemes, but the device described is a start.  With some modifications, I bet

you could route the output to a dumb terminal (or thru a null modem cable) in

order to READ the data.  Worth checking out the schematics.

As for making your own cards, just paste a length of VCR, reel-to-reel, or

audio cassette tape to a cut-out posterboard or plastic card.  Works just as

good as the real thing, and useful to experiment with if you have no expired or

'dead' ATM or calling cards lying around (SAVE them, don't TOSS them!).

** Examples of Data on Magstripes **

The real fun in experimenting with magstripe technology is READING cards to

find out WHAT THE HELL is ON them!  Haven't you wondered?  The following cards

are the result of my own 'research'.  Data such as specific account numbers and

names has been changed to protect the innocent.  None the cards used to make

this list were stolen or acquired illegally.

Notice that I make careful note of 'common data'; data that I noticed was

the same for all cards of a particular type.  This is highlighted below the

data with asterisks (*).  Where I found varying data, I indicate it with "x"'s.

In those cases, NUMBER of CHARACTERS was consistent (the number of "x"'s equals

the number of characters...one to one relationship).

I still don't know what some of the data fields are for, but hopefully I

will be following this phile with a sequel after I collect more data.  It ISN'T

easy to find lots of cards to examine. Ask your friends, family, and co-workers

to help!  "Hey, can I, um, like BORROW your MCI calling card tonite?  I'm

working on an, um, EXPERIMENT.  Please?"  Just...be honest!  Also, do some

trashing.  People will often BEND expired cards in half, then throw them out.

Simply bend 'em back into their normal shape, and they'll usually work (I've

done it!).  They may be expired, but they're not ERASED!

-------------------------------------------------------------------------------

-=Mastercard=-  Number on front of card -> 1111 2222 3333 4444

Expiration date -> 12/99

Track 2 (BCD,75 bpi)-> ;1111222233334444=99121010000000000000?

***

Track 1 (ALPHA,210 bpi)-> %B1111222233334444^PUBLIC/JOHN?

*

Note that the "101" was common to all MC cards checked, as well as the "B".

-------------------------------------------------------------------------------

-=VISA=-  Number on front of card -> 1111 2222 3333 4444

Expiration date -> 12/99

Track 2 (BCD,75 bpi)-> ;1111222233334444=9912101xxxxxxxxxxxxx?

***

Track 1 (ALPHA,210 bpi)-> %B1111222233334444^PUBLIC/JOHN^9912101xxxxxxxxxxxxx?

*

Note that the "101" was common to all VISA cards checked, as well as the "B".

Also, the "xxx" indicates numeric data that varied from card to card, with no

apparent pattern.  I believe this is the encrypted pin for use when cardholders

get 'cash advances' from ATMs.  In every case, tho, I found *13* digits of the

stuff.

-------------------------------------------------------------------------------

-=Discover=-  Number on front of card -> 1111 2222 3333 4444

Expiration date -> 12/99

Track 2 (BCD,75 bpi)-> ;1111222233334444=991210100000?

********

Track 1 (ALPHA,210 bpi)-> %B1111222233334444^PUBLIC/JOHN___^991210100000?

********

Note, the "10100000" and "B" were common to most DISCOVER cards checked.

I found a few that had "10110000" instead.  Don't know the significance.

Note the underscores after the name JOHN.  I found consistently that the name

data field had *26* charaters.  Whatever was left of the field after the name

was "padded" with SPACES.  Soo...for all of you with names longer than 25

(exclude the "/") charaters, PREPARE to be TRUNCATED! ;)

-------------------------------------------------------------------------------

-=US Sprint FON=-  Number on front of card -> 111 222 3333 4444

Track 2 (BCD,75 bpi)-> ;xxxxxx11122233339==xxx4444xxxxxxxxxx=?

*

Track 1 (ALPHA,210 bpi)-> %B^ /^^xxxxxxxxxxxxxxxxx?

*

Strange.  None of the cards I check had names in the Track 1 fields.  Track 1

looks unused, yet it was always formatted with field separators.  The "xxx"

stuff varied from card to card, and I didn't see a pattern.  I know it isn't

a PIN, so it must be account data.

-------------------------------------------------------------------------------

-=Fleet Bank=-  Number on front of card -> 111111 222 3333333

Expiration date -> 12/99

Track 2 (BCD,75 bpi)-> ;1111112223333333=9912120100000000xxxx?

****

Track 1 (ALPHA,210 bpi) ->

%B1111112223333333^PUBLIC/JOHN___^9912120100000000000000xxxx000000?

*                                    ****

Note that the "xxx" data varied.  This is the encrypted PIN offset.  Always 4

digits (hrmmm...).  The "1201" was always the same.  In fact, I tried many

ATM cards from DIFFERENT BANKS...and they all had "1201".

-------------------------------------------------------------------------------

(Can't leave *this* one out ;)

-=Radio Shack=-  Number on front of card -> 1111 222 333333

NO EXPIRATION data on card

Track 2 (BCD,75 dpi)-> ;1111222333333=9912101?

*******

Note that the "9912101" was the SAME for EVERY Radio Shack card I saw.  Looks

like when they don't have 'real' data to put in the expiration date field, they

have to stick SOMETHING in there.

-------------------------------------------------------------------------------

Well, that's all I'm going to put out right now.  As you can see, the major

types of cards (ATMs, CC) all follow the same rules more or less.  I checked

out a number of security passcards and timeclock entry cards..and they ALL had

random stuff written to Track 2.  Track 2 is by FAR the MOST utilized track on

the card.  And the format is pretty much always ANSI/ISO BCD.  I *did* run

into some hotel room access cards that, when scanned, were GARBLED.  They most

likely used a character set other than ASCII (if they were audio tones, my

reader would have put out NOTHING...as opposed to GARBLED data).  As you can

see, one could write a BOOK listing different types of card data.  I intended

only to give you some examples.  My research has been limited, but I tried to

make logical conclusions based on the data I received.

** Cards of All Flavors **

People wanted to store A LOT of data on plastic cards.  And they wanted that

data to be 'invisible' to cardholders.  Here are the different card

technologies that were invented and are available today.

HOLLERITH - With this system, holes are punched in a plastic or paper card and

read optically.  One of the earliest technologies, it is now seen

as an encoded room key in hotels.  The technology is not secure,

but cards are cheap to make.

BAR CODE -  The use of bar codes is limited.  They are cheap, but there is

virtually no security and the bar code strip can be easily damaged.

INFRARED -  Not in widespread use, cards are factory encoded by creating a

"shadow pattern" within the card.  The card is passed thru a swipe

or insertion reader that uses an infrared scanner.  Infrared card

pricing is moderate to expensive, and encoding is pretty secure.

Infrared scanners are optical and therefore vulnerable to

contamination.

PROXIMITY - Hands-free operation is the primary selling point of this card.

Altho several different circuit designs are used, all proximity

cards permit the transmission of a code simply by bringing the card

near the reader (6-12").  These cards are quite thick, up to

0.15" (the ABA standard is 0.030"!).

WIEGAND -   Named after its inventor, this technology uses a series of small

diameter wires that, when subjected to a changing magnetic field,

induce a discrete voltage output in a sensing coil.  Two rows of

wires are embedded in a coded strip.  When the wires move past

the read head, a series of pulses is read and interpreted as binary

code.  This technology prodces card that are VERY hard to copy

or alter, and cards are moderately expensive to make.  Readers

based on this tech are epoxy filled, making them immune to weather

conditions, and neither card nor readers are affected by external

magnetic fields (don't worry about leaving these cards on top of

the television set...you can't hurt them!).  Here's an example of

the layout of the wires in a Wiegand strip:

||| || ||   | ||| | || ||    |  ||  ||    |   |  ||

|  |    | |   | |     ||||     ||  ||||     ||

The wires are NOT visible from the outside of the card, but if

your card is white, place it in front of a VERY bright light source

and peer inside.  Notice that the spacings between the wires is

uniform.

BARIUM FERRITE - The oldest magnetic encoding technology (been around for 40

yrs!) it uses small bits of magnetized barium ferrite that are

placed inside a plastic card.  The polarity and location of

the "spots" determines the coding.  These cards have a short

life cycle, and are used EXTENSIVELY in parking lots (high

turnover rate, minimal security).  Barium Ferrite cards are

ONLY used with INSERTION readers.

There you have the most commonly used cards.  Magstripes are common because

they are CHEAP and relatively secure.

** Magstripe Coercivity **

Magstripes themselves come in different flavors.  The COERCIVITY of the

magnetic media must be specified.  The coercivity is the magnetic field

strength required to demagnetize an encoded stripe, and therefore determines

the encode head field strength required to encode the stripe.  A range of media

coerciviteis are available ranging from 300 Oersteds to 4,000 Oe.  That boils

down to HIGH-ENERGY magstripes (4,000 Oe) and LOW-ENERGY magstripes (300 Oe).

REMEMBER: since all magstripes have the same magnetic remanence regardless of

their coercivity, readers CANNOT tell the difference between HIGH and LOW

energy stripes.  Both are read the same by the same machines.

LOW-ENERGY media is most common.  It is used on all financial cards, but its

disadvantage is that it is subject to accidental demagnetization from contact

with common magnets (refrigerator, TV magnetic fields, etc.).  But these cards

are kept safe in wallets and purses most of the time.

HIGH-ENERGY meda is used for ID Badges and access control cards, which are

commonly used in 'hostile' environments (worn on uniform, used in stockrooms).

Normal magnets will not affect these cards, and low-energy encoders cannot

write to them.

** Not All that Fluxes is Digital **

Not all magstripe cards operate on a digital encoding method.  SOME cards

encode AUDIO TONES, as opposed to digital data.  These cards are usually

used with old, outdated, industrial-strength equipment where security is not an

issue and not a great deal of data need be encoded on the card.  Some subway

passes are like this.  They require only expiration data on the magstripe, and

a short series of varying frequencies and durations are enough.  Frequencies

will vary with the speed of swiping, but RELATIVE frequencies will remain the

same (for instance, tone 1 is twice the freq. of tone 2, and .5 the freq of

tone 3, regardless of the original frequencies!).  Grab an oscilliscope to

visualize the tones, and listen to them on your stereo.  I haven't experimented

with these types of cards at all.

** Security and Smartcards **

Many security systems utilize magstripe cards, in the form of passcards and

ID cards.  It's interesting, but I found in a NUMBER of cases that there was

a serious FLAW in the security of the system.  In these cases, there was a

code number PRINTED on the card.  When scanned, I found this number encoded on

the magstripe.  Problem was, the CODE NUMBER was ALL I found on the magstripe!

Meaning, by just looking at the face of the card, I immediately knew exactly

what was encoded on it.  Ooops!  Makes it pretty damn easy to just glance at

Joe's card during lunch, then go home and pop out my OWN copy of Joe's access

card!  Fortunately, I found this flaw only in 'smaller' companies (sometimes

even universities).  Bigger companies seem to know better, and DON'T print

ALL of the magstripe data right on card in big, easily legible numbers.  At

least the big companies *I* checked. ;)

Other security blunders include passcard magstripes encoded ONLY with the

owner's social security number (yeah, real difficult to find out a person's

SS#...GREAT idea), and having passcards with only 3 or 4 digit codes.

Smartcard technology involves the use of chips embedded in plastic cards,

with pinouts that temporarily contact the card reader equipment.  Obviously,

a GREAT deal of data could be stored in this way, and unauthorized duplication

would be very difficuly.  Interestingly enough, not much effort is being put

into smartcards by the major credit card companies.  They feel that the tech

is too expensive, and that still more data can be squeezed onto magstripe

cards in the future (especially Track 1).  I find this somewhat analagous to

the use of metallic oxide disk media.  Sure, it's not the greatest (compared

to erasable-writable optical disks), but it's CHEAP..and we just keep improving

it.  Magstripes will be around for a long time to come.  The media will be

refined, and data density increased.  But for conventional applications, the

vast storage capabilities of smartcards are just not needed.

** Biometrics: Throw yer cards away! **

I'd like to end with a mention of biometrics: the technology based on reading

the physical attributes of an individual thru retina scanning, signature

verification, voice verification, and other means.  This was once limited to

government use and to supersensitive installations.  However, biometrics will

soon acquire a larger market share in access control sales because much of its

development stage has passed and costs will be within reach of more buyers.

Eventually, we can expect biometrics to replace pretty much ALL cards..because

all those plastic cards in your wallet are there JUST to help COMPANIES

*identify* YOU.  And with biometrics, they'll know you without having to read

cards.  I'm not paranoid, nor do I subscribe to any grand 'corporate

conspiracy', but I find it a bit unsettling that our physical attributes will

most likely someday be sitting in the cool, vast electronic databases of

the CORPORATE world.  Accessable by anyone willing to pay.  Imagine CBI and TRW

databases with your retina image, fingerprint, and voice pattern online for

instant, convenient retrieval.  Today, a person can CHOOSE NOT to own a credit

card or a bank card...we can cut up our plastic ID cards!  Without a card, a

card reader is useless and cannot identify you.  Paying in cash makes you

invisible!  However, with biometrics, all a machine has to do is watch...

.listen...and record.  With government/corporate America pushing all the

buttons..  "Are you paying in cash?...thank you...please look into the camera.

Oh, I see your name is Mr. Smith...uh, oh...my computer tells me you haven't

paid your gas bill....afraid I'm going to have to keep this money and credit

your gas account with it....do you have any more cash?...or would you rather

I garnish your paycheck?"  heh heh

** Closing Notes (FINALLY!!!!) **

Whew...this was one MOTHER of a phile.  I hope it was interesting, and I hope

you distribute it to all you friends.  This phile was a production of

"Restricted Data Transmissions"...a group of techies based in the Boston area

that feel that "Information is Power"...and we intend to release a number of

highly technical yet entertaining philes in the coming year....LOOK FOR THEM!!

Tomorrow I'm on my way to Xmascon '91.....we made some slick  buttons

commemorating the event...if you ever see one of them (green wreath..XMASCON

1991 printed on it)..hang on to it!...it's a collector's item.. (hahahah)

Boy, I'm sleepy...

Remember....    "Truth is cheap, but information costs!"

But -=RDT is gonna change all that... ;)  set the info FREE!

Peace.

..oooOO Count Zero OOooo..

count0@world.std.com

count0@spica.bu.edu

count0@atdt.org

ATDT------(???)YOU-WISH

(You're not paranoid if they're REALLY out to get you... ;)