Freeview Videocrypt Decoding Program Infomation Page

Freeview 2.1 is the same kind of program as season, which initially appeared

at utelscin.el.utwente.nl, which also works with the Pace-Receivers which

keeped locking up the decoder after about 5 minutes, making the user swap

channels, or remove and insert the card back into the decoder.

If you start Freeview without any parameters, Freeview should behave the same

way SEASON used to. If you run Freeview with a new Pace-Decoder, you may have

to specify another timing (wc) to avoid decoder lockups. Values around 150000

have been reliable in conjunction with a 386DX-33 Notebook and a Pace MSS500.

When you specified this timing, you will notice two things:

- The computer screen will alternativly display 'RESET'

and 'Decrypting xxx (xx xx xx xx)'.

If you read a stable 'Decrypting...' - message, increase the value in wc.

- The decoder onscreen messages won't disappear.

This can easily be fixed by using Freeview's m-option to supress

any onscreen displays.

There may be problems, when the interface's power is supplied from the

decoder; if this version doesn't work, when there's a value for wc set, use

an external 5 volt power supply to the interface.

Post any suggestions please to

alt.satellite.tv.europe

or

alt.satellite.tv.crypt

Commandline syntax: FREEVIEW [com][wax][wbx][wcx][wdx][wex][m][e][d][u]

com: comport nr e.g. 1   wc - we: experimental only

x is a numeric value     m: suppress OSD display  i:do not use Interupts

wa: delay time for reset e: no PC screen updates

wb: byte delay time      d: start in debug mode

u:  load in user names

Possible keyboard commands while the program is running:

q exit the program    a increase reset delay   1 increase proc. delay

d toggle debug mode   b decrease reset delay   2 decrease proc. delay

l write logfile       + increase byte delay    n toggle nano debug mode

c change com port     m toggle OSM             u use user channel names

s save user channels

NOTES

-----

wa thru we are delays, all entered in ms (milliseconds)

wa wait between reset and answer to reset

wb wait after each outgoing byte

wc wait between header and first procedure byte

wd wait between procedure byte and data

we wait between final data byte and procedure bytes

Example: FREEVIEW 1 wa0 wb0 m

starts FREEVIEW using comport 1 with reset delay and byte delay set at

0 milliseconds, and with onscreen display suppressed.

NEW OPTION USER CHANNEL NAMES

After writing a few friends different version of freeview, that have

different channels naem in, for one resason, or another, I have

included a way for anyone to change the screen messages.  All you

need to do is use a test editor and edit the file called CHANNELS.TXT,

please make sure the you do not change the lenght longer than they are

in the file, Spaces must be either '.' or '*' the program will convert

these into spaces for you, DO NOT USE SPACES IN FILE, THIS WILL NOT

WORK IF YOU DO. If the file is missing run freeview and then press

the 'S' or 's' key this will create a blank file for you to edit.

Hope you like this.

Version History

V2.1:  Option to not use interupt as timer, this should allow you to

run in windows, and on 8086 computers.

NEW OPTION to use you own channel name by changing a text file,

FREEVIEW will then load in this file.

V2.0:  Display name of multi channel,

New option to select com port from program,

Online help screen,

Toggle screen message on or off,

Internal code rewrote.

V1.8:  Fixes all ECM's upto 23 September 1995

V1.5:  Fixes the Nanocommand 28 used by sky form 29 June 1995

V1.4:  Fix the new ECM for Sky 15-June 1995.

V1.3:  Refix for Adult Channel

V1.2:  Fixes the Perry Smith ECM on both Adult Channels

V1.1:  Fixes to BSkyB ECMs from 6-Apr-95,

Fixes to the nasty "No key fits"-display problem,

Some internal cleaning-up done done earlier.

Fixes BSkyB ECM from 8-Mar-95 (shame,shame!)

(internal) Fixes Adult Channel ECM from mid-Feb;

TV Eurotica Channel ID implemented.

Adult decoding included

To Download the latest version of Freeview just look for the file FREEVIEW.ZIP

by

FTP

ftp.gpl.net/paulmax/freeview.zip

ftp.gpl.net/paulmax/tron/freeview.zip

WWW

http://www.gpl.net/paulmax/index.hmtl

http://www.gpl.net/paulmax/tron/freeview.html

All the Best

Tron